SAFETY DATA SHEETAFT Silicone Free Lubricant 500ml

SECTION 1; IDENTIFICATION OF THE SUBSTANCES/MIXTURE AND OF THE COMPANY/UNDERTAKING

  - Product identifier

Product name                         NON SIL

Product No.                             ADH NON-SIL 500

  - Relevant identified uses of the substances or mixture and uses advised against

Identified uses                        Spray lubricant

  - Details of the supplier of the data sheet

Supplier                           	AFT Aerosols Ltd

Unit 8, Berryhill industrial estate

Berryhill road

Fenton

Stoke- on- Trent

ST4 2NL

  - Emergency telephone number

01782 285 700

SECTION 2; HAZARDS IDENTIFICATION

2.1 Classification of the substance or mixture

Classification (EC 1272/2008)

Physical and chemical hazards    Extremely Flam. Aerosol- H222

Environment                                  Not Classified

The full text for all Hazard statements are displayed in Section 16.

2.2 Label Elements

Label in Accordance with (EC) No. 1272/2008

Signal word		    Danger

Hazard statements

H222                       Extremely flammable aerosol.

H229		        Pressurized container: may burst if heated

Precautionary Statements

P102                        Keep out of reach of children

P210                        Keep away from heat/sparks/open flames/hot

surfaces – No

Smoking.

P251                        Pressurized container: Do not pierce or burn, even

after use.

P280		        Wear protective gloves/protective clothing/eye

protection/face protection.

P501                        Dispose of contents/container in accordance with

Local Regulations.

Supplementary precautionary statements

P273                       Avoid release to the environment.

P264                       Wash contaminated skin thoroughly after handling.

P304+340              IF INHALED: Remove victim to fresh air and keep at

rest In a position comfortable for breathing.

P305+351+338      IF IN EYES: Rinse cautiously with water for several

minutes. Remove contact lenses, if present and easy

to do.Continue Rinsing.

P308+313               If exposed or concerned: Get medical advice/attention

P403                        Store in a well-ventilated place

P410+412               Protect from sunlight. Do not expose to temperatures

Exceeding 50°C/122°F.

2.3. Other hazards

Pressurised container. Protect from sunlight and do not expose to temperatures exceeding 50°C.

Do not pierce or burn even after use. Do not spray on naked flame or any incandescent material –

NO SMOKING.

SECTION 3: COMPOSITION/INFORMATION ON INGREDIENTS

3.2 Mixtures

VEGETABLE OIL 5-10% CAS-No.: Not Classified EC No.: Not Classified

Classification (EC 1272/2008) Not Classified

PETROLEUM GASES LIQUIFIED 30-60% CAS- No.: 68476-85-7 EC No.: 270-704-2

Classification (EC 1272/2008) Flam. Gas 1- H220

The full text for all hazard statements are displayed in Section 16.

SECTION 4: FIRST AID MEASURES

4.1 Description of first aid measures

General information

Move the exposed person to fresh air at once. Get medical attention if any discomfort continues.

Inhalation

Move the exposed person to fresh air at once.  When breathing is difficult, properly trained personnel may assist affected person by administering oxygen. Keep the affected person warm and at rest.   Get prompt medical attention.

Ingestion

DO NOT induce vomiting. Get medical attention immediately

Skin contact

Wash the skin immediately with soap and water. Promptly remove clothing if soaked through and wash as above.  Get medical attention if any discomfort continues.

Eye Contact

Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to do. Continue

rinsing.

4.2 Most important symptoms and effects, both acute and delayed

4.3 Indication of any immediate medical attention and special treatment needed

SECTION 5: FIREFIGHTING MEASURES

5.1 Extinguishing media

Extinguishing media

Fire can be extinguished using: foam; carbon dioxide; dry powder

5.2 Special hazards arising from the substance or mixture

Unusual fire & Explosion hazards

Aerosol cans may explode in fire.

5.3 Advice for firefighters

Wear self contained breathing apparatus for fire fighting if necessary.

SECTION 6: ACCIDENTAL RELEASE MEASURES

6.1. Personal precautions, protective equipment and emergency procedures

Avoid skin and eye contact. Ensure adequate ventilation. Wear personal protective equipment.

6.2 Environmental precautions

Avoid discharge into the environment if possible.

6.3 Methods and material for containment and cleaning up

Provide ventilation and confine spill. Do not allow runoff to sewer. Absorb in vermiculite, dry sand or earth, and place into containers.

6.4 Reference to other sections

Wear protective clothing as described in section 8 of this safety data sheet.

SECTION 7: HANDLING AND STORAGE

7.1 Precautions for safe handling

Keep away from heat, sparks and open flame. Avoid spilling, skin and eye contact. Ventilate well, avoid breathing vapours. Use approved respirator if air contamination is above accepted level.

7.2. Conditions for safe storage, including any incompatibilities

Aerosol cans: Must not be exposed to direct sunlight or temperatures above 50°C.

7.3 Specific end use(s)

The identified uses for this product are detailed in Section 1.2.

SECTION 8: EXPOSURE CONTROLS/PERSONAL PROTECTION

8.1 Control parameters

| Name | STD | TWA – 8 Hrs |  | STEL – 15 Min |  | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| PETROLEUM GASES LIQUIFIED | WEL | 1000 ppm (Sk) | 1250 mg/m3(Sk) | 1250 ppm(Sk) | 2180 mg/m3(Sk) |  |

Ingredient comments

WEL = Workplace exposure limits.

8.2 Exposure controls

Protective equipment

Appropriate engineering controls

Observe any occupational exposure limits for the product or ingredients. As this product contains ingredients with exposure limits, process enclosures, local exhaust ventilation or other engineering controls should be used to keep worker exposure below any statutory or recommended limits, if use generates dust, fumes, gas, vapour or mist.

Eye/face protection

Chemical splash goggles or face shield. Use equipment for eye protection tested and approved under appropriate government standards such as EN 166(EU).

Hand protection

Considering the data specified by the glove manufacturer, check during use that the gloves are retaining their protective properties and change them as soon as any deterioration is detected. Nitrile rubber.

Other skin and body protection

Wash hands and any other contaminated areas of the body with soap and water before leaving the work site. Wear protective clothing.

Respiratory protection

If ventilation is inadequate, suitable respiratory protection must be worn. Respiratory protection complying with an approved standard should be worn if a risk assessment indicates inhalation of contaminants is possible. Use respirators and components tested and approved under appropriate government standards such as CEN (EU).

Hygiene measures

DO NOT SMOKE IN WORK AREA! Wash hands at the end of each work shift and before eating, smoking and using the toilet. Promptly remove any clothing that becomes contaminated. When using do not eat, drink or smoke.

SECTION 9: PHYSICAL AND CHEMICAL PROPERTIES

9.1 Information on basic physical and chemical properties

(a) Appearance					Aerosol.

(b) Odour					Hydrocarbon

c) Odour Threshold				No data available

(d) pH						No data available

(e) Melting point/freezing point		No data available

(f) Initial boiling point and boiling range	40 (°C)

(g) Flash point					Estimated at -35ᴼC

(h) Evaporation point				No data available

(i) Flammability (solid gas)			No data available

(j) Upper/lower flammability

Or explosive limits				No data available

(k) Vapour pressure				No data available

(l) Vapour density				No data available

(m) Relative density				No data available

(n) Water solubility				No data available

(o) Partition coefficient

n-octanol/water				No data available

(p) Auto-ignition temperature			No data available

(q) Decomposition temperature		No data available

(r) Viscosity					No data available

(s) Explosive properties				No data available

(t) Oxidising properties 				No data available.

9.2 Other information

SECTION 10: STABILITY AND REACTIVITY

10.1 Reactivity

10.2 Chemical stability

Avoid heat, sparks, and flames, stable under normal conditions.

10.3 Possibility of hazardous reactions

10.4 Conditions to avoid

Avoid heat, flames and other sources or ignition. Avoid contact with: Strong oxidising agents, Strong alkalis and Strong mineral acids.

10.5 Incompatible materials

Materials to avoid

Strong acids, strong oxidising substances and strong alkalis.

10.6 Hazardous decomposition products

Fire creates: Toxic gases/vapours/fumes of: Carbon monoxide (CO); Carbon Dioxide (CO2)

SECTION 11: TOXICOLOGICAL INFORMATION

11.1 Information on toxicological effects

Other Health Effects

No data available

Inhalation

Harmful by inhalation

Ingestion

May cause discomfort if swallowed.

Skin contact

Prolonged contact may cause dryness of the skin. May cause sensitisation by skin contact

Eye contact

Spray and vapour may cause Irritation and smarting

.

SECTION 12: ECOLOGICAL INFORMATION

Ecotoxicity

Not regarded as dangerous to the environment. However, contamination of the aquatic or terrestrial environments should be avoided

12.1 Toxicity

No data available

12.2 Persistence and degradability

No data available

12.3 Bio accumulative potential

No data available

12.4 Mobility in soil

No data available

12.5 Results of PBT and vPvB Assessment

No data available

12.6 Other adverse effects

No data available

SECTION 13: DISPOSAL CONSIDERATIONS

13.1 Waste treatment methods

Empty containers must not be burned because of explosion hazard. Dispose of waste and residues in accordance with local authority requirements.

SECTION 14: TRANSPORT INFORMATION

14.1 UN Number

UN No. (ADR/RID/ADN)             1950

UN No.  (IMDG)                           1950

UN No.  (ICAO)                             1950

14.2 UN proper shipping name

Proper shipping name – AEROSOLS

14.3 Transport hazard class(es)

ADR/RID/ADN class          2.1

ADR/RID/ADN class          Class 2: Gases

ADR Label No                    2.1

IMDG Class                        2.1

ICAO Class/division          2.1

ICAO Subsidiary risk	 None

CEFIC TEC® No		 20GSF

Air Class		 2.1

UK Road Class		 2.1

Transport labels	 L.Q.

14.4 Packing group

Not applicable.

14.5 Environmental hazards

Environmentally hazardous substance/marine pollutant.

No.

14.6 Special precautions for user

EMS                                               F-D, S-U

Tunnel restriction code             (D)

14.7 transport in bulk according to Annex II of MARPOL73/78 and the IBC code.

SECTION 15: REGULATORY INFORMATION

15.1 Safety, health and environmental regulations/legislation specific for the substance or mixture:

Labelling according to Regulation (EC) No 1272/2008

The chemicals (Hazard information and packaging for supply) regulations 2009 (S.I 2009 No. 716). Control of substances hazardous to health.

Approved code of practice.

Guidance notes

Workplace exposure limits EH40.

15.1.1 EU-Regulations

Contains no REACH substances with Annex XVII restrictions.

Contains no REACH Annex XIV substances.

15.2 Chemical safety assessment

SECTION 16: OTHER INFORMATION

General information

This product should be used as directed. For further information consult the product data sheet or contact technical services.

Information sources

This safety data sheet was compiled using current safety information supplied by distributor raw materials.

Revision comments

This safety data sheet supersedes all previous issues and users are cautioned to ensure that it is current. Destroy all previous data sheets and if in doubt contact AFT Aerosols Ltd.

Hazard statements in full

H220                       	Extremely flammable gas

H222		Extremely flammable aerosol

ISSUE:	REV 4.2

REVISION DATE:			JANUARY 2016

DISCLAIMER

The Information provided herein, especially recommendations for the usage and the application of this products, is provided in good faith, and no liability on the part of AFT Aerosols Ltd is stated or implied.  No employee of AFT Aerosols Ltd has the authority to waive or alter in any way the content of this document.

Due to different materials used, as well as to varying working conditions, production techniques, and the requirements of the end users, all of which are beyond our control, we strongly recommend that thorough and extensive trials are carried out in order to test the suitability of our products with regard to the required processes and applications. This should also include an ageing test which should be applied to all substrates used.

It is also the responsibility of the purchaser and end user of this product to ensure that all appropriate actions necessary for the protection of the environment, and for the health and safety of their employees are observed.

This datasheet replaces all former versions